- Shelter in Place (SIP)

- Shelter in Place Procedure

Shelter in place is a public protection tool during hazardous materials accidents to minimize the exposure of the threatened public to dangerous chemical(s). Sheltering in place uses a structure and its indoor atmosphere to temporarily separate people from a hazardous outside atmosphere.

Some exposure will occur, but if properly undertaken, sheltering in place can provide substantial protection from doses high enough to cause injury.

In the event that a shelter in place is advised for the campus, all persons will be notified by the Security Department in conjunction with Administration and EH&S.

- How to Shelter in Place – Building

- Close and lock all doors and windows and other opening to the outside.

- Shut off the buildings ventilation system (HVAC).

- Security post signs on doors – “Shelter in Place in Effect – NO ENTRY.”

- If possible, move to interior room(s) above ground floor level with fewest windows and vents.

- If possible, use plastic sheeting and duct tape to seal all cracks around doors and any vents in the room.

- DO NOT LEAVE or open the building for any reason until an All Clear is given by Security or the Unified Command Center, Public Information Officer (UCC PIO)

- How to Shelter in Place – Vehicle

- If you are close to home or public building, go there immediately and go inside.

- If you are unable to, pull over to the side of the road. Stop your vehicle in the safest place possible.

- Turn off engine and close windows and vents.

- If possible, seal the heating/AC vents with duct tape.

- Listen to the radio regularly for updated advice and instructions.

- Stay where you are until you are told it is safe to get back on the road.